University of Cairo

Faculty of Engineering

Department of Computer Engineering

Third Year

Report

Project Title :

Simulation of Circuit Switching Communication systems

Project Team :

Ahmed Adel Sobeih

Ahmed Mohammed Reda

Mohammed Assem Abdou

Supervised by :

Prof Dr Talaat Salem El-Sheikh

Course :

Communication systems Engineering

1997-8    First semester

1. Introduction :

It is a truth universally acknowledged that simulation programs are one of the most important computer applications today . A few decades ago , an engineer had to build the system he designed , or model its structure if he could , so as to get insight into its behaviour , judge it or even understand it . However , a designer can now, thanks to computers , use the powerful mathematical and analytical capabilities of simulation programs in order to study the behaviour of his design without actually implementing it . Moreover , he can easily modify the parameters of his design so as to see the effect of each of them on the whole system . He can also obtain statistics and produce reports .

This document is a report on a program that simulates circuit switching communication systems . The program is implemented by the students whose names appear on the cover page .

The document starts with an abstract of the program , followed by a scientific background to circuit switching ( if you do not know what    TS , ST and TST systems are , you are strongly urged to read that section ) . The design of the program is then fully explained together with the results obtained . There is also a user guide that explains how you can use the program . Finally our projected plans and conclusions are documented .

We do hope that you really enjoy and avail yourselves of our program .

2. Abstract :

This program simulates the behaviour of TS , ST and TST circuit switching systems ( if you do not understand these terms , or any following ones , you can later refer to the background section ) . We have thought of simulating these three systems in one program so that we can compare their responses under similar conditions and hence judge each of them .

Particular mention must be made of calculating the probability of blocking    which is one of the important criteria in judging a circuit switching communication system .

The program enables you to start a new simulation , pause or end a running simulation , continue a paused simulation or trace a simulation one step at a time . You can also obtain result files showing the sequence of call arrivals , how the system responds to every call and the probability of blocking . You can save your network settings in a network file and later open it when needed .

A final conclusion is then made concerning which of the three systems has turned out to be the best in most circumstances .

3. Acknowledgement

A    project such as this would not have come about without the help and encouragement of some very special people . Above all , we would like to offer our heartfelt thanks with a respectful handshake    to Eng. Amgad Zeitoun (a graduate assistant in the department of computer engineering)    for all the support and backing he provided us with. He was very helpful , co-operative , kind and tactful.    He always kept his promises , showed true devotion and gave generously of his time and expertise .

To name a    few of his services , he was the first to tell us what multithreading is , and how to build an MDI program with multiple document templates . Besides , he guided us to various books and Internet sites that really helped us develop this project .

Thanking him and feeling especially indebted to him , we wish him all the best and do hope that one day we will be able to equally reward him (if an equal reward is ever possible).

Ahmed Adel Sobeih

Ahmed Mohammed Reda

Mohammed Assem Abdou

4. Software package and System Requirements :

Software Package :

The software package used in developing this project is Microsoft Visual C++ version 4.0 under Windows 95 .

Software Requirements :

The program is intended to run in the Windows 95 environment. All what you have to do is to double-click the executable file name (simulate.exe) from the Windows Explorer.

Hardware Requirements :

If you have a 100 MHz Pentium with a 16 MB RAM , you are in a very good position to run the program . However , seeing that the program is multithreading and has extensive graphics , larger memories and/or higher speeds are recommended .

5. Background :

5.1 Circuit Switching :

Communication via circuit switching implies that there is a dedicated communication path between stations. That path is a connected sequence of links between network nodes. On each physical link , a logical channel is dedicated to the connection. This path is reserved until the communication session ends. Data    generated by the source station are transmitted along the dedicated path as rapidly as possible. At each node, incoming data are routed or switched to the reserved outgoing channel without delay .

5.2 Applications of Circuit Switching :

Circuit switching was developed to handle voice traffic but is now also used for data traffic. The best known example of a circuit-switching network is the public telephone network; this is actually a collection of national networks interconnected to form the international service. Another well-known application of circuit switching is the private branch exchange (PBX), used to interconnect telephones within a building or office.    A final common example of the application of circuit switching is the data switch. The data switch is similar to the PBX but is designed to interconnect digital data-processing devices, such as terminals and computers .

5.3 Blocking :

An important characteristic of a circuit-switching device is whether it is blocking or nonblocking. Blocking occurs when the network is unable to connect two stations, even if the called party is free, because all possible paths between the two stations are already in use.    A blocking network is one in which such blocking is possible . The probability of blocking is the ratio between blocked calls to the total number of initiated calls. This probability is usually obtained over a long period of time, including high-traffic hours, so as to be an economically sound figure of merit. On the other hand, a nonblocking network permits all stations to be connected (in pairs) at once and grants all possible connection requests as long as the called party is free. When    a    network    is    supporting    only    voice    traffic,    a blocking configuration is generally acceptable, as it is expected that most phone calls are of short duration and that therefore only a fraction of the telephones will be engaged at any time. However, when data processing devices are involved, these assumptions may be invalid. For example, for a data-entry application, a terminal may be continuously connected to a computer for hours at a time. Hence, for data applications, there is a requirement for a nonblocking or “nearly nonblocking” (very low probability of blocking) configuration.

5.4 The single-stage crossbar switch :

A simple space switch matrix suitable for use with N inlets and M outlets is shown in Fig. 1 .

Outlets

1              2          3                                                                                                    M

1

2

Inlets

N

Figure 1 . The single-stage crossbar switch

The basic building block of the crossbar switch is a metallic crosspoint or semiconductor gate at each inlet-outlet intersection. This crosspoint can be enabled and disabled by a control unit. Communication is possible between an inlet and outlet by enabling the appropriate crosspoint. For example, the above figure shows a crossbar switch with two connections set up: 1 with 3 and 2 with M .

It can be seen that that the switch is nonblocking ; a path is always available to connect an inlet to an outlet, as long as the outlet is free.

It should be noted that the single-stage crossbar switch is quite costly since the number of crosspoints grows with the square of the number of attached stations. Another alternative is to employ multiple-stage switches. However, the multiple-stage switch may be blocking.

5.5 Time (T) Switching ( Time Slot Interchange ) :

The basics of time switching by time-slot interchanging is illustrated in Fig. 2 .

Figure 2 Illustrating the principle of time-slot interchanging

At the transmitting side, a multiplexer (and an A/D converter) multiplexes and encodes signals from N subscribers. At the receiving end, a demultiplexer carries out the inverse process. The multiplexer and demultiplexer switches maintain a fixed synchronism so that the switches contact terminals 1 at the same time, terminals 2 at the same time, etc.

The xth    word in the slot content memory contains all the bits that constitute the input time slot (sample) data obtained from the xth inlet. Now suppose that the xth inlet wants to connect to the yth outlet, then the number x itself is written in the yth word of the control memory. This control memory provides addresses for reading from the slot content memory. Hence, during the yth time slot, the yth word of the control memory is read. Recall    that    this    word    contains x, and since this is the address for reading from the slot content memory, the xth word of the slot content memory is read and sent to the yth output. Therefore, x is effectively connected to y. Fig. 2 shows a time-slot interchanger (T switch) with the following connections: 7 to 1, N to 2, 18 to 3,and 8 to N.

A time-slot interchanger (TSI) system has the merit of providing full availability and is also nonblocking.

5.6 Space (S) Switching :

The use of a space switch matrix to provide some switching is shown in Fig. 3.

Figure 3 A space array used to provide switching of digital signals

Here we have assumed an equal number of inlets and outlets. A number N, of T-slot multiplexers, provide and the outlets are connected to N, T-slot demultiplexers. There is no slot interchange between multiplexers and demultiplexers. Thus when the multiplexers are connected to slot k the demultiplexers are also connected to slot k. The gate select memory has T locations, that is, as many locations as there are slots. The address for reading the memory is a counter which increments at the slot rate. The content of the word so read out of the memory is the information about which gates of the space matrix are to be enabled in the corresponding slot interval. The translator decodes the word and provides the corresponding enabling activation for the gates. In each column of the space matrix only one gate will be enabled at a time. Thus, for example, it might be that during the interval of slot 1 : inlet 1 might be connected to outlet 7, inlet 2 to outlet 9, etc. During slot 2, it might be that inlet 1 is connected to outlet 4, inlet 2 to outlet 5, etc. Observe again that the space matrix provides no time-slot interchange. It simply moves the content of time-slot k on one inlet to time-slot k on any (available) outlet.

It is possible for the space switching system shown above to provide as many as NT simultaneous connections. However, it suffers seriously from the handicap that it provides only very poor accessibility. Connections are possible from slot k only to slot k. Thus slot 1 on inlet 1 can be connected to N slot 1 outlets while N(T-1) other outlets are not accessible. As a result, Space (S) switching is not usually employed alone, but it is usually used with a TSI system.

5.7 Time-Space (TS) Switching :

Fig. 4 shows a time-space switching system.

Figure 4 Time-Space (TS) switching

Here we have a space matrix with N inlets and N outlets. For each inlet line we have introduced a time slot interchanger with T slots. Time slot memories have to be provided for each interchanger and a gate select memory needs to be provided for the space array. These memories are not shown in the figure. A subscriber is assigned a time slot, and a line of the space array. Thus suppose then that communication is to take place between subscriber A assigned time slot 3 and line 8 and subscriber B assigned time slot 5 and line 12. Then the signal generated by A is moved from time slot 3 to time slot 5 by the time slot interchanger and is transferred from line 8 to line 12 in the space matrix. Similarly the signal originated by B is moved from slot 5 to slot 3 and from line 12 to line 8.

The number of simultaneous connections possible with the arrangement of the time-space switch is NT. For example, if T = 128 and if the space matrix is a 16 * 16 matrix, i.e., N = 16, then 128 * 16 = 2048 connections can be supported. However, the structure is not free of blocking. If we considered the above example of A and B again, and suppose further that subscriber C assigned time slot 2 and line 8 wants to connect to subscriber D assigned time slot 5 and line 11; the signal generated by C can NOT be moved from time slot 2 to time slot 5 by the TSI of line 8 because time slot 5 already contains the signal generated from A, and thus blocking would occur, and C can not get through to D although D is free. Generally speaking, if two subscribers X and Y    using different time slots on the same inlet line want to connect to two subscribers Z and W using the same time slot on different outlet lines, blocking would occur.

5.8 Space-Time (ST) Switching :

An alternate arrangement to the time-space switching consists of moving the time slot interchangers to the outlet side of the space array. This forms an ST switch. The two arrangements are equally effective; that is, the probability of blocking is expected to be nearly the same. However, in this case, blocking would occur if    two subscribers X and Y    using the same time slot on    different inlet lines want to connect to two subscribers Z and W using    different time slots on the same outlet line.

5.9 Time-Space-Time (TST) Switching :

An elaboration of the TS and ST systems is shown below. Here a space array is sandwiched between sets of time-slot interchangers yielding a TST switching arrangement as shown in Fig. 5 .

Figure 5 Time-Space-Time (TST) switching

This scheme provides more alternative paths and hence, while blocking is still possible, the likelihood of blocking is reduced. To see the relative merit of the TST scheme over a TS (or ST) scheme, let us start with the TST arrangement except with the input T stages omitted so that we have an ST arrangement. Suppose that in this ST switch a connection is already established between time slot 5 on input line 7 and time slot 13 on output line 9. This means that during time slot 5, input line 7 is connected to output line 9. During time slot 5, the contents of slot 5 is transferred from inlet 7 to outlet 9. The interchanger on outlet 9 will move the content of slot 5 to slot 13. Now suppose further that while this first connection is in effect, there is a need to connect slot 5 on inlet line 8 to slot 14 on outlet line 9. The point is that there is a need to transfer two time slots 5, on different inlets, to different slots on the same outlet. It is clear that a block will occur against the second proposed cobecause during slot 5 the outlet line is busy. If, however, we also have time slot interchangers on the input side of the space matrix as shown above, then the connection in the space matrix need not be made in slot 5. Instead the input interchanger can move the content of slot 5 to some other slot, say slot 6 such that the outlet line is not busy during that slot 6, and thus the transfer from inlet to outlet of that second proposed connection can be accomplished in that time slot (slot no. 6). The output interchanger will now be called upon to move slot 6 to slot 14. Thus the slot interchange from 5 to 14 will take place in two stages, from 5 to 6 and then from 6 to 14. Blocking will only occur if and only if during all the empty (unused) slots of the TSI of the input line, the output line is busy (i.e. not available, already connected to another input line); which is a much smaller probability than that of TS or ST switching.

5.10 References :

1. William Stallings , Data and Computer Communications ,                                      Fifth Edition , Prentice-Hall International, Inc.

- Herbert    Taub      and      Donald    L.    Schilling      ,      Principles of

Communication Systems , Second Edition .

- Andrew S. Tanenbaum, Computer Networks , Third Edition,

Prentice-Hall International, Inc.

6. Program Design

6.1 Classes and Data Structures :

This section explains the structure of the classes and data structures implemented in the program. The specification is written in C++; however, it should be easily understandable by readers with knowledge of any other similar high-level programming language

class CConnection

{    // This class implements a connection between two subscribers.

//    It specifies the calling party, the called one, the call duration

//    and when the call has been initiated.

protected :

// Data members

int    m_InputLine              ;          // inlet of a connection

int    m_InputSlot                ;          // slot number on an inlet

int    m_OutputLine        ;          // outlet of a connection

int    m_OutputSlot          ;          // slot number on an outlet

unsigned long    m_Duration ;

// duration of the call in slots

unsigned long    m_Start        ;

// the time at which the call begins given in slots

// e.g. if m_start = 1600

// then the call begins at t = 1600 slots

// measured from the start of simulation

public :

// Member functions

CConnection() ;        // class default constructor

void SetConnection(    int input_line    , int input_slot	 ,

int output_line , int output_slot ,

unsigned long duration , unsigned long start) ;

// SetConnection() sets the values of the data members.

void SetStart(unsigned long lStart) ;

// SetStart() sets the start of a call.

CConnection( int input_line    , int input_slot	 ,

int output_line , int output_slot ,

unsigned long start ,    unsigned long duration)

// non-default constructor

int GetInputLine()    { return m_InputLine    ; }

int GetOutputLine() { return m_OutputLine ; }

int GetInSlot()          { return m_InputSlot    ; }

int GetOutSlot()        { return m_OutputSlot ; }

unsigned long GetDuration()        { return m_Duration ; }

unsigned long GetCallStart()      { return m_Start ; }

};

/////////////////////////////////////////////////////////////////////////////////////////////////

class CST_TSI

{ // This class implements the Time Slot Interchanger of    an ST

//    network topology .

// Data Members

int* to ;                              // time slot interchanger

int m_Size ;                  //    size of the interchanger ; this is the number

//    of words in the slot content memory

public :

// Member Functions

CST_TSI(int T) ;            // class constructor

void SetValue(int index, int value);

// SetValue() sets a slot value in the TSI

// If a slot value is -1, then that slot is not used ,

// otherwise the slot value is the line to which the slot goes

int    GetValue(int index);

// GetValue() gets a slot value from the TSI

int    CheckBusyReceiver(int key);

// CheckBusyReceiver() checks if the receiver of the current call

// is busy or free .

~CST_TSI();

//class destructor that deallocates the memory of the “to” array

};

/////////////////////////////////////////////////////////////////////////////////////////////////

struct ST_Call

{    // This simple structure stores the outlet and duration of a call

int outlet ;      unsigned long duration ;

} ;

/////////////////////////////////////////////////////////////////////////////////////////////////

class CST_GateSelect

{ // This class implements the gate select memory of the ST

// network topology

// Data Members

ST_Call** memory ;      // a pointer to the gate select 2D array

// Each entry in the gate select 2D array stores an ST_Call

// instance variable; hence, stores the outlet and duration of

// the corresponding call

int m_Lines ;                              //    number of columns of the 2D array

int m_Slots ;                                //    number of rows of the 2D array

public :

// Member Functions

CST_GateSelect(int T , int N) ;        // class constructor

void SetOutlet(int row, int column, int out);

int    GetOutlet(int row, int column);

void SetDuration(int row, int column, unsigned long hold_time);

unsigned long GetDuration(int row, int column);

int CheckDuplicateTransmitter(int Line, int Slot) ;

// Since the transmitter and receiver of a call are randomized

// during simulation, we must check if the transmitter is still

// engaged in a previous call .

~CST_GateSelect();

// class destructor that deallocates the memory of the 2D array

};

/////////////////////////////////////////////////////////////////////////////////////////////////

CTimeSwitching:

This class maps for a single time switch in the TST switching . There is a single member function in that class which is m_control_array. This is a pointer to a list of integers. It is used to determine when the samples in the input frame will be in the output frame . This list is created such that    any element m_control_array[i] carries the input time slot that will be output at slot number i .

The constructor of this class dynamically allocates the space for the m_control_array list and initiates the whole list with the value NOT_USED to indicate that initially nothing is used in the control array .

The destructor of this class simply deallocate the memory assigned for the m_controlý_array list .

CSpace:

This class maps for the space switch in the TST switching. There are two member variables for keeping the number of inlets and the number of slots, Also there is a pointer to a list of pointers to calls. This pointer is used to dynamically-allocate space for a two dimensional list of call structures to keep the contents of the gate select memory during simulation .

The constructor for this class dynamically allocates the space for the gate select memory and initiates the whole list with the value NOT_USED to indicate that initially nothing is used .

The destructor of this class simply deallocate the memory assigned for the gate select memory list .

A member function named “CheckDuplicateTransmitter” is	 included in the class. This function is used during call randomization to make sure that the new call is not coming from a transmitter who is already busy.

Note :

Seeing the fact that the TSI and gate select classes of one network topology are very similar to those of the other two topologies, a good decision is to implement a generic base class for the TSI and another one for the gate select. The classes of each topology are then derived from these base classes, where each derived class modifies or adds the functionality that suits the corresponding topology.

After specifying the classes that store the data of the calls and the switches, we now proceed to explain the document and view classes. In fact, these two classes are essential to most Visual C++ programs. The document class is responsible for storing the program data as well as for reading and writing this data to disk files. The view class is responsible for displaying the program data (on the screen, printer, or other device) and for processing input from the user.

This program is an MDI (Multiple Document Interface) application. This means that you are allowed to open several documents at a time; each document is viewed and edited within a separate child frame window that is contained within the application workspace of the program’s main frame window.

Moreover, the program has multiple document types - a “network” document, and a “results” document. You can open or create a document (file) of either type , and documents of the two types can be opened and manipulated simultaneously.

The “results” document is basically a document that stores ASCII text characters (derived from CEditView). That document stores the results of a simulation. Due to its simplicity and readability, the “results” document is not explained any further. If you still need to know more about the “results” document refer to its header and implementation files (those starting with CResults ).

However, we will fully explain the “network” document because it is indispensable to the understanding of    the whole program.

CSimulateDoc : public CDocument

{

protected:

// Data members

int m_N ;                        // number of inlets(outlets) of the space array

int m_T ;                          // number of time slots

// Number of input(output) subscribers is N*T

int m_interval ;

// simulation interval ; this determines the simulation speed

Topology m_type ;

// the last type of circuit switching topology

// used for this document(i.e. file of a certain network)

// For a new file , this is initialised as TST switching)

unsigned long m_ncalls ; // number of calls(connections)

FileStatus m_status ;

// this indicates whether the file is newly-created

// or have been previously saved and loaded.

int m_AveCallDuration ;

// the average number of seconds that a call can last for .

int m_AveArrivalRate ;

// the average number of calls initiated per minute

int m_MaxOneCallPer ;

// this means that a subscriber does at most 1 call

// per m_MaxOneCallPer minutes...

unsigned int m_Seed ;

// m_Seed stores the seed of the random number generator

BOOL m_MenuFlag ;

// m_MenuFlag is used to disable some menu items that are

// meaningless to be enabled during simulation ,

// e.g. The user can not change the parameters of a network

// whose operation is currently being simulated...

// Member functions

CSimulateDoc();    // class constructor

int RNG( int lower_limit , int upper_limit );

// RNG() is used to generate a pseudorandom integer

// from lower_limit to upper_limit .

unsigned long RandomizeNewCall (CConnection& call) ;

// RandomizeNewCall() is used to randomize a new call

Topology GetTopology()    { return m_type ; }

int GetT() { return m_T ; }

int GetN() { return m_N ; }

unsigned int GetSeed() { return m_Seed ; }

unsigned long GetNumCalls() {return m_ncalls;}

void SetInitialDrawArea();

public:

// Member Functions

void EnableMenuFlag( BOOL flag 	{ m_MenuFlag = flag ; }

int GetSimulationInterval()	{ return m_interval ; }

};

/////////////////////////////////////////////////////////////////////////////////////////////////

class CSimulateView : public CScrollView

{

protected:

CSimulateView();			// Constructor

// Attributes

// specifies the step used in zooming in and out

float m_ZoomingRatio;

// The following declarations belong to the TST simulator:

CSpace *m_S;						//	 S

CTimeSwitching **m_T[2];				// T	                T

int TST_ServiceCall();

private:

void WriteFrameInfo(void);

void LineWRTRect(int x1,int y1,int x2,int y2,CDC *pDC);

// Draws a line using the m_ZoomingRatio			void RectangleWRTRect(int x1,int y1,int x2,int y2,CDC*pDC);

// Draws a rectangle using the m_ZoomingRatio		void DrawTime(int x,int y,int col,int index);

// Draws a time switch

void UpdateTimeElement(int x,int y,int col,int line,int

elementno,CDC *pDC);

// Updates the time switch

void UpdateTimeElementForAllViews(int x,int y,int col,int line,int elementno);

void UpdateGSElementForAllViews(int x,int y,int input,int slot);

void UpdateGSElement(int x,int y,int input,int slot,CDC

*pDC);

// Update an element in the gate select memory

void DrawSpace(int x,int y);

// Draws the Space Switch

void DrawMUX(int x,int y);

// Draws a Multiplexer

void DrawDEMUX(int x,int y);

// Draws a Demultiplexer

void DrawGS(int x,int y);

// Draws the gate select memory

protected:

SimulationStatus m_SimulationStatus ;

// a pointer to the array of TSI

CST_TSI** m_StTsi ;

// GateSelect signifies the control memory of the space array

CST_GateSelect* m_StGateSelect ;

// a pointer to the array of TSI of TS topology

CTS_TSI** m_TsTsi ;

// GateSelect signifies the control memory of the space array

CTS_GateSelect* m_TsGateSelect ;

int m_CurrentSlot ;    // current slot in simulation

unsigned long m_CurrentFrame ;

unsigned long m_CallCount ; // holds a counter of the number of calls that have been simulated

CConnection m_CurrentCall ; // holds the current call used in simulation

int m_MoreCalls ; // a flag used to indicate whether there are new calls to simulate or not

int m_NewCall ;      // a flag used to indicate whether a new call has arrived or not

int m_ValidCall ; // a flag used to indicate whether the current new call is valid or not

unsigned long m_BlockNum ;

CWinThread* m_pWinThread ;

int m_Threading ; // a flag used to indicate whether the simulation thread

// has ever been initiated

int    m_YesFile ; // =1 if the user wants to output results to file

CStdioFile* m_pFile ;

char m_OutputFileSpec [MAX_FILE_SPEC] ;

// the path and name of the results file;

BOOL m_FrameAndSlot ; // =TRUE if the user wants to output the frame and slot number

BOOL m_SrcAndDest ; // =TRUE if the user wants to output the source and destination of a call

int m_CMContents ;

// output the frame contents every

//      frame      >>      m_CMContents = 0

//      second    >>      m_CMContents = 1

//      minute    >>      m_CMContents = 2

//      never      >>      m_CMContents = 3

public:

CSimulateDoc* GetDocument();

void Zoom(int in_or_out);

// in_or_out = 1    >>> Zoom in

// in_or_out = -1 >>> Zoom out

CRect GetDimRectangle(long left,long top,long right,long bottom) ;

void DrawLine(CDC* pDC , int x1, int y1 , int x2 , int y2 ) ;

unsigned long MOD(unsigned long Dividend ,

unsigned long Divisor ) ;

// CheckNewCallArrival() sees if the current call has arrived or not yet

int CheckNewCallArrival(unsigned long Start) ;

// ST_ServiceCall() services a call if the topology is ST

// Servicing means determining whether :

//      >>> the receiver is busy          >>> return ST_BUSY

//      >>> the call is blocked            >>> return ST_BLOCKED

//      >>> the call is non-blocked    >>> return ST_NONBLOCKED

int ST_ServiceCall();

// ST_UpdateGateSelect() updates the duration of each call in the gate select

// AND if a call terminates it removes its data from the gate select

void ST_UpdateGateSelect() ;

void TST_UpdateGateSelect() ;

// EndSimulation() is used to end simulation

void ST_EndSimulation() ;

void TST_EndSimulation() ;

void ST_OutputGateSelect();

void TST_OutputGateSelect();

int TS_ServiceCall();

void TS_UpdateGateSelect() ;

void TS_EndSimulation() ;

void TS_OutputGateSelect();

friend UINT SimulationThread(LPVOID pParam) ;

void PrepareFirstCall() ;

void TerminateSimulation() ;

// Operations

public:

// Overrides

public:

virtual void OnDraw(CDC* pDC);    // overridden to draw this view

virtual BOOL PreCreateWindow(CREATESTRUCT& cs);

protected:

virtual void OnInitialUpdate(); // called first time after construct

virtual BOOL OnPreparePrinting(CPrintInfo* pInfo);

virtual void OnBeginPrinting(CDC* pDC, CPrintInfo* pInfo);

virtual void OnEndPrinting(CDC* pDC, CPrintInfo* pInfo);

// Implementation

public:

virtual ~CSimulateView();

};

/////////////////////////////////////////////////////////////////////////////

6.2 Call Randomization and Initiation :

This section explains how the calls are randomized  during simulation.

On setting a network parameters, the user specifies the average duration of a call ( t ) in seconds, and the average arrival rate (lambda calls per minute ) . The program simulates the response of the circuit-switching network to the calls initiated in an hour.

The sampling rate is 8000 samples/second; hence the time of one frame is 125 microseconds. It is also assumed that each of the T slots making one frame has an equal slot duration to that of all the other slots, and with no gaps between channels. Hence, the duration of one slot (sample) is 125/T microseconds.

6.2.1 Call Duration :

The duration of a call is randomly chosen for every call to be in the range : t/2    to 3t/2    seconds.

Seeing that the sampling frequency is 8000 samples per second, a call lasting for y seconds (where t/2 < y < 3t/2) would require y*8000 samples; that is y*8000 frames where each frame contains one sample (slot) of that call.

6.2.2 Call Initiation :

The calls arrive according to the average arrival rate which is given in lambda calls per minute. Hence, the average time between two successive calls is 60/lambda seconds. If the average number of slot durations between two successive calls is x , then

x    *    125 microsec / T = 60 / lambda

Hence,

x    =      ( T * 60 * 8000 ) / lambda .

For every call initiated, the number of slots between that call and the one just preceding it is randomly chosen to be in the range : x/2 to 3x/2. Let us call this number d. If we add d to the start of the preceding call, we obtain the time at which the new call arrives.

6.3 Multithreading :

The term thread    refers to the execution of a sequence of program instructions. The main thread in a program is called the primary thread. A program written for Windows 95 or Windows NT can start one or more secondary threads, each thread independently executing a sequence of instructions within the program code. From the viewpoint of the user or application programmer, the threads in a program run simultaneously. The operating system typically achieves this apparent simultaneity by rapidly switching control from thread to thread (if the computer has more than one processor, however, the system can execute threads literally simultaneously).

Multithreading is especially useful in a Windows GUI program, where the primary program thread can be dedicated to processing messages so that the program can respond quickly to commands and other events. A second thread can be used to perform any lengthy task that would block program message processing if performed by the primary thread.

The circuit switching simulator program contains two threads - a primary thread and a secondary thread. The second thread is dedicated to simulation, i.e. processing call arrivals, writing the sequence of events to the results file, and drawing graphics. Hence, during simulation, you can do whatever you want : select a menu item , resize the view window , or perhaps pause or end the simulation (thread).

The simulation thread is started whenever you choose to start a new simulation. The function AfxBeginThread() is used to start a new thread. The third parameter passed to this function specifies the priority of the thread. In simple terms, the priority of a thread determines how frequently the thread is allowed to run as the operating system switches control from thread to thread. The priority level that showed the best performance was THREAD_PRIORITY_HIGHEST .

The thread is suspended (paused) whenever you select the Pause Simulation menu item , and resumed whenever you select Continue Simulation.

6.4 Graphics

The program draws the simulated circuit in a view window.    A zooming facility is added to allow the user to have either an overall view or a detailed view.

This requires the graphics in the program to be sizable . To achieve this a member variable named m_ZoomingRatio was included with each view.    This ratio is multiplied by any coordinate which is going to be used to draw on the screen .

In order not to complicate the parts in the program which access the screen, the m_ZoomingRatio variable wasn’t used everywhere in the program, but it was only used in certain functions and the rest of the program use these functions. These functions are substitutes for the normal base graphics functions offered by the MFC like (LineTo, Rectangle, etc ) . The same naming convention was used but some characters were added to differentiate them from the original MFC functions.

Functions are written to draw each of the Circuit Switching elements like the TSIs and the gate select memories.

There are other functions dedicated to updating the circuit elements. When the simulation is running , these functions are called and the contents of the TSIs and the gate select memories are updated.

When the user selects to zoom in(out) the view window, the m_ZoomingRatio variable is increased(decreased) and the updating functions are called to redraw the view .

8. Results and Conclusions:

As was previously mentioned, the program outputs the sequence of calls arrival, together with the results of simulation to a “results” file. The results file has the extension (nwr); you can open and read a “results” file from within the program itself.

A “results” file typically contains the frame and slot numbers at which a call arrives, the source and destination of this call, its duration, whether the call has been established or blocked or the called party was busy, and the contents of the gate select memory. The following listing is an excerpt of a typical “results” file showing the results of an ST simulation. The gate select memory contents are written to the “results” file every minute.

T = 8 and N = 10

No. of calls = 62

Frame:0 ,Slot:0.A new call has arrived.

Callno . 0 .

Line 7 Slot 7 wants to call Line 3 Slot 6.

The call would last for 26.00 sec.

The call is established.

0          1          2          3            4          5          6          7            8            9

0        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

1        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

2        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

3        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

4        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

5        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

6        -1        -1        -1          7        -1        -1        -1        -1        -1        -1

7        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

Frame:235 ,Slot:0.A new call has arrived.

Call no . 1 .

Line 7 Slot 0 wants to call Line 4 Slot 1.

The call would last for 37.00 sec.

The call is established.

0            1          2          3            4          5          6          7            8            9

0        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

1        -1        -1        -1        -1          7        -1        -1        -1        -1        -1

2        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

3        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

4        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

5        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

6        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

7        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

Frame:527 ,Slot:4.A new call has arrived.

Call no . 2 .

Line 5 Slot 4 wants to call Line 1 Slot 1.

The call would last for 47.00 sec.

The call is established.

Frame:746 ,Slot:4.A new call has arrived.

Call no . 3 .

Line 4 Slot 7 wants to call Line 2 Slot 7.

The call would last for 50.00 sec.

The call is established.

0          1          2          3            4          5          6          7            8            9

0        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

1        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

2        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

3        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

4        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

5        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

6        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

7        -1        -1          4        -1        -1        -1        -1        -1        -1        -1

Frame:1033 ,Slot:0.A new call has arrived.

Call no . 4 .

Line 6 Slot 6 wants to call Line 2 Slot 7.

The call would last for 38.00 sec.

The called party is busy.

Frame:1361 ,Slot:0.A new call has arrived.

Call no . 5 .

Line 1 Slot 4 wants to call Line 4 Slot 7.

The call would last for 50.00 sec.

The call is established.

0            1          2          3            4          5          6          7            8            9

0        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

1        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

2        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

3        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

4        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

5        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

6        -1        -1        -1        -1        -1        -1        -1        -1        -1        -1

7        -1        -1        -1        -1          1        -1        -1        -1        -1        -1

Frame:1502 ,Slot:4.A new call has arrived.

Call no . 6 .

Line 4 Slot 2 wants to call Line 9 Slot 3.

The call would last for 28.00 sec.

The call is established.

Frame:1655 ,Slot:4.A new call has arrived.

Call no . 7 .

Line 5 Slot 0 wants to call Line 6 Slot 4.

The call would last for 60.00 sec.

The call is established.

Frame:1815 ,Slot:4.A new call has arrived.

.

.

.

.

.

Frame:14584 ,Slot:4.A new call has arrived.

Call no . 61 .

Line 1 Slot 6 wants to call Line 5 Slot 2.

The call would last for 45.00 sec.

The call is established.

Simulation terminated

Number of blocked calls = 1 out of 62 calls .

Conclusions

After we have obtained several “results” files for the three different network topologies, we have found out that :

1 . The probability of blocking was nearly the same for the ST and TS topologies.

2 .    The TST topology had a much lower probability of blocking than that of the other two topologies.

3 . In the three different topologies, increasing the average arrival rate and/or the average duration of a call increases the probability of blocking.

9. Unfulfilled Plans and Impressions :

Although we have made all what was required to do in this project, there were some plans that we could not fulfil because of time limitations, and    the standstills which the project came to due to examinations and other preoccupations.

The major unfulfilled plans are :

1. Not only a simulation, but also a tutorial : It has been proposed to include a tutorial that can be used to teach the basics of space and time circuit switching, so that students can use it to understand the TS, ST and TST network topologies.

- Backward simulation :  The project allows you to step through the simulation one step forward. However, we have also thought to give you the added capability of going backward.

- Printing and Print-Previewing : Although the project offers printing and print-previewing the results file, it would have been better if you can also print and print-preview the network diagram with the TSIs and gate select memories.

Finally, we would like to declare that developing this project was really useful and fruitful; we had a very good chance to practise VC++ in a way that has extremely improved our Windows-programming skills. We would like to thank Prof Dr Talaat El-Sheikh, and do hope that the students of the next year have a similar project assignment.

10. How to contact us ? :

Here are our phone numbers and e-mail addresses, so that you can contact us whenever you need . All your questions are confidential, and will be replied in three days’ time .

Ahmed Adel Sobeih

Phone : 4304874

E-mail : esobaih@alpha1-eng.cairo.eun.eg

Ahmed Mohammed Reda

Phone : 3855232

E-mail : mlatif@ritsec1.com.eg

Mohammed Assem Abdou

Phone : 5940666

11. References :

1. Michael J. Young , Mastering Microsoft Visual C++ 4 ,                                      Second Edition , Sybex Inc.

2.      David J. Kruglinski    ,    Inside Visual C++ 4 , Microsoft Press.

3.      James L. Conger , Windows API Bible , Waite Group Press .

- Charles Petzold, Programming Windows 3.1, Microsoft Press.